GENERATOR PROCEDURE

PLEASE FOLLOW CAREFULLY – THIS THING CAN

HURT OR KILL YOU AND/OR THE RADIOS

# Starting

- Unlock the padlock with the key you were given. Do NOT run the generator in the laundry room – you will be a DEAD carbon monoxide victim. Roll it out onto the ground about 6 feet away from the house. Let the wheels go out and down first and then the rest carefully over the ledge and step.

- Add fuel if necessary. Gas is stored in the pile step closet. Do NOT store fuel in the house or laundry room. Check the oil level and add oil if necessary. 10W-30 is in the laundry room on the back shelf.

- Unroll the yellow extension cords (only one if you will be running barefoot) that are on the shelf in the laundry room and route them around the west end of the house, across the porch, and into the shack.  One is for 110 and one for 220.

- Be SURE the power switches on all equipment in the shack are OFF!

- Check that the yellow cords are NOT plugged into the generator.

- Turn on the fuel valve – it’s in the rubber line under the tank. Be gentle. Pull the starter cord gently a couple of times, turn on the red switch on the engine housing, set the choke, and pull-start the engine. Reset the choke for smooth operation, then plug in the yellow cords at the generator.

- Unplug the red plug from the wall box and plug it into the 110 volt outlet box on the yellow cable, from the generator. Begin turning the gear back on VERY conservatively. Keep the load on the generator low.

- If you MUST have linears, plug them into the 220 volt yellow Romex cable from the generator, and run them very conservatively.

- Do NOT refuel the generator while it is running or while it is hot.

- When the commercial power comes back on, put the amp(s) in standby, give them at least five minutes to cool down, then turn them off. Turn off all power switches on the transceivers and computers. IMMEDIATELY turn the amp outboard fan back on so that the linear does not shock-cool. Re-plug the red plug into its red-taped box on the wall, and the linears into the plastic wiring duct outlets where they were before, and resume the contest.

# Stowing

- Roll up the yellow cords and wipe them off so that the next user has a clean and neat start.

- Run the generator until the tank is completely dry and it dies on its own.

- Tug the generator back into the laundry room, wheels last, and relock the chain to the plumbing.

- Be sure there is no gas in the tank of the generator.

- Turn off the fuel valve.

- Put the fuel can back in the step closet.