WinKeyer

We use an external  WinKeyer box to handle our CW and PTT chores. The logging program off loads CW messages to be sent to WinKeyer in the form of ASCII characters and the WinKeyer takes care of converting them to perfect CW rig keying, as well as doing the PTT with the correct amount of lead and tail time for the amplifiers' relays. This relieves the Windows operating system in the computer from the over head of having to try and produce quality CW.

The latest WinKeyer firmware is v.30.

Fours of our newest WinKeyers can be updated via the USB cable if need be.

The others need a firmware chip replacement.

Physical connections

The WinKeyer USB lead goes to USB port on the computer. It helps to avoid creating extra virtual serial ports o the computer by always plugging the WinKeyer into the same USB port each time.

The WinKeyer CW Keying lead goes to the K3's KEY socket. Male RCA plug to male 1/4” plug.

The WinKeyer PTT lead goes to the AMP's PTT in socket. Male RCA plug to male RCA plug.

N1MM+ Logger's WinKeyer tab in the configuration menu sets the appropriate lead/tail times  and over-rides any settings native to the WinKeyer. In N1MM+ Logger you can also tell the WinKeyer to run the paddle at a different speed (usually slower) then the canned CQ/exchange messages using the speed knob on the front of the WinKeyer box.

Please see the N1MM+ Logger set up section of this manual for the gory details of configuring the WinKeyer.